- 4 Megapixel CMOS Sensor

- Onvif Profile S Compliant

- 3-Axis adjustment

- 2.8-12mm Motorised Lens

- 3D Digital Noise Reduction

- Digital Wide Dynamic Range

- Power over Ethernet

- 20-30m IR LED

- True Day/Night IR Cut Filter

- H.265 / H.264 Compression

- Dual Streaming

- Micro-SD Card Slot

- Grey or White Housing

PRIME IP4-EEDI2812MG/W

External Eyeball Dome with Motorised Zoom Lens (4MP)

| Hardware |  |
| --- | --- |
| Image Sensor | 1/3” Sony Exmor CMOS |
| Electronic Shutter | 1/25s ~ 1/10000s |
| Sensitivity | 0 Lux (IR On) 0.5 Lux Colour / 0.2 Lux B/W |
| Lens Type | 2.8 - 12mm Motorised AF Lens |
| Lens Interface | M12 |
| ICR | Mechanical IR cut filter |
| LED | 2 Hi-Intensity IR LED 20-30m Distance (scene dependent) |
| Alarm I/O | No |
| PoE | Yes, (IEEE802.3af) |
| Connections | 12VDC, RJ45 Network |
| Power | PoE 802.3af / 12VDC 5.5W (IR Off), 7W (IR On) |
| Operating Temperature | -10ºC ~ 50ºC IP65 Rated |
| Dimensions | ø120mm x 95mm (H) |
| Weight | 680g |

| System |  |
| --- | --- |
| Video Resolution options | 2592 x 1520 @ 20pps At 25pps: 2560 x 1440, 1080P, 720P, D1, VGA, 2CIF, CIF, QCIF |
| Video Adjustment options | Brightness, Contrast, Saturation, Sharpness, AGC, WDR, Mirror, 3D Noise reduction, Defog |
| Dual Streaming | Yes |
| Image Snapshot | Yes |
| Privacy Mask | Yes - 4 zones |
| Compression Format | H.265 / H.264 / MJPEG |
| Video Bitrate range | 16Kbps ~ 8Mbps |
| Onboard Storage | Micro-SD Card Slot |
| Video Bitrate Encoding | CBR, VBR |
| Motion Detection | Yes - Multi level sensitivity |
| OSD | Time/Date, Title |
| Alarm Functions | Video Loss, Network, Tampering, Motion Detection, ROI |
| Trigger Actions | Email, Snapshot, Zoom Preset |
| Security | Password |
| Firmware Upgrade | Yes, possible remotely via browser |

Technical Specification

| Network |  |
| --- | --- |
| Ethernet | 10/100 Base-T RJ45 cable connection |
| Network Protocols | TCP/IP, UDP, HTTP, DHCP, NTP, RTSP, PPPoE, DDNS, SMTP, UPNP, FTP |

Acam Technology Ltd, 3 Prime Parkway, Derby DE1 3QB - www.acamtechnology.com Tel 01332 382555 - Fax 01332 382666

(Specifications subject to change without notice)